Roll no: 241701021

Experiment 9

Aim:

To design and configure a simple network consisting of a router, switch, access point, and multiple wired and wireless end devices, and to verify communication between them.

Components Required:

- 1 Switch

- 2 PC

- 1 Laptop

- 1 Access point

- 1 Tablet

- Copper straight through

Procedure:

- Add Devices

  - Place the following devices in the workspace:

    - 1 Router (Router0)

    - 1 Switch (Switch1)

    - 1 Access Point (Access Point0)

    - 1 Wired PC (PC0)

    - Wireless devices: PC1, Laptop0, TabletPC0

- Connect Devices (Wired)

  - Connect PC0 → Switch1 using a straight-through cable.

  - Connect Switch1 → Router0 using a straight-through cable.

  - Connect Switch1 → Access Point0 using a straight-through cable.

- Configure Router

  - Assign an IP address to the router interface (e.g., 192.168.1.1).

  - Enable the interface.

- Configure Switch

  - No major configuration required (default settings are sufficient).

  - Ensure all ports are active.

- Configure Access Point

  - Set SSID (e.g., "WirelessLAN").

  - Configure security (WPA2-PSK with a password).

  - Assign IP address if required (or use DHCP).

- Configure End Devices

  - PC0 (wired):

    - Assign IP manually (e.g., 192.168.1.2, subnet mask 255.255.255.0, gateway 192.168.1.1)

  - Wireless Devices (PC1, Laptop0, TabletPC0):

    - Connect to the SSID created on the access point.

    - Enter password.

    - Assign IP manually or via DHCP (e.g., 192.168.1.3–192.168.1.5)

- Verify Connectivity

  - Use the ping command:

    - From PC0 to Router0

    - Between wired and wireless devices

    - Between wireless devices

Observation:

fig. Final circuit

Result:

The network was successfully created and configured. All wired and wireless devices were able to communicate with each other.